BiPAP Synchrony Ventilatory Support System

The BiPAP Synchrony provides efficient and comfortable non invasive ventilation for chronic respiratory insufficiency patients.

BiPAP Synchrony provides Respironics advanced technology:

- Digital Auto-TRAK™ Sensitivity provides optimized trigger setting, that automatically adapt to changes in leaks and changes in patient respiratory mechanics

- AVAPS feature guarantees the safety of an averaged tidal volume maintained while providing the comfort of a pressure support mode of ventilation

Vent Ramp and RiseTime also aid in the delivery of comfortable ventilation.

Compliance therapy and trends data collected from the SmartCard and then managed by software help clinicians assess home ventilation effectiveness and adjust settings to maintain comfort and efficiency of therapy in the long term. Synchrony® is also compatible with  polygraph to provide detailed information on patients’ ventilation on a breath-to-breath basis.

Traveling is easy with the Synchrony, as it weighs only 1,8 kilograms and can be connected to an external battery with a 7 hrs autonomy at EPAP/IPAP of 4/20cm H2O and 12 bmp.

Specifications

Ventilation Modes

CPAP, Spontaneous (S), Spontaneous/Timed (S/T), Timed (T), Pressure Control (PC), AVAPS feature

Settings

| AVAPS |  |
| --- | --- |
| IPAP min IPAP max Target Vte Alarm | EPAP to IPAP max IPAP min to 30 cmH2O 200 ml to 1500 ml Low Vte (can be disabled) |
| IPAP | 4 to 30 cmH2O |
| EPAP | 4 to 25 cmH2O |
| CPAP | 4 to 20 cmH2O |
| Breath Rate | 0 to 30 BPM (PC and S/T) 4 to 30 BPM (T) |
| Inspiratory Time | 0.5 to 3.0 sec |
| Rise Time | 100 to 600 msec (1 to 6) |
| Vent Ramp | 0 to 45 min. |

Physical

| Dimensions | 24 cm (L) x 17 cm (W) x 11 cm (H) |
| --- | --- |
| Weight | 1,8 Kilograms |

Electrical

| AC Voltage | 100 to 240 V, 50/60 Hz |
| --- | --- |
| DC Voltage | 12 V |
| AC Current | 1.25 A Maximum |
| DC Current | 5.5 A Maximum |

Trigger and Cycle

| Triggering | Automatic triggers with Auto-Trak algorithm: - Shape Signal - Volume 6 cc above baseline |
| --- | --- |
| Cycling | Automatic cycling with Auto-Trak algorithm: - Spontaneous Expiratory Threshold (SET) - Shape Signal - Maximum Inspiratory Time of 3.0 sec - Flow Reversal |

Accessories

| External battery | 7 hrs autonomy at EPAP/IPAP of 4/20cmH2O and 12 bmp |
| --- | --- |
| Integrated humidifier | 5 levels of heating |
| Compliance software | Encore Pro software and Encore SmartCard, can be connected to Stardust II polygraph |