BiPap® Vision®

Ventilatory Support System

This versatile, simple, noninvasive ventilation system is designed with advanced technology to be accurate, reliable and easily upgradeable.

The BiPAP Vision System offers a low cost alternate form of sub-acute therapy for patients and may eliminate the need for intubation in appropriate candidates. The integrated display screen uses easy-to-read real time graphics in waveforms or bar scale format to provide enhanced monitoring for caregivers. Auto-Trak SensitivityT, ensures optimum sensitivity despite changing breathing patterns or circuit leaks.

CE marked to the Medical Device Directive Caution: U.S. federal law restricts these devices to sale by or on the order of a physician.

BiPap® Vision®

Specifications

Modes

| Mode | Control Range | Control Increments |
| --- | --- | --- |
| CPAP | 4 to 20 cm H2O | 1 cm H2O |
| S/T | 4 to 40 cm H2O | 1 cm H2O |
| PAV * /T | Quick Start Menu | Obstructive, Restrictive, Mixed, Normal |

*Proportional Assist Ventilation. Not available for sale in the US.

Control Ranges & Increments: Parameters

| Parameters | Control Range | Control Increment |
| --- | --- | --- |
| IPAP | 4 to 40 cm H2O | 1 cm H2O |
| EPAP | 4 to 20 cm H2O | 1 cm H2O |
| CPAP | 4 to 20 cm H2O | 1 cm H2O |
| Rate | 4 to 40 BPM | 1 BPM |
| Timed Inspiration | 0.5 to 3.0 sec | 0.1 sec |
| IPAP >Rise Time | 0.05 to 0.40 sec | 4 set points: .05, .1, .2, .4 sec |
| Oxygen Concentration (%O2)* | 21 to 100% | 4% from 21 to 25% 5% from 25 to 100% |

*With optional Oxygen Module

Control Ranges & Increments: Alarms

| Alarm | Control Range | Control Increment |
| --- | --- | --- |
| High Pressure | 5 to 50 cm H2O | 1 cm H2O |
| Low Pressure | Disabled;0 to 40 cm H2O | 1 cm H2O |
| Low Pressure | Delay 0 to 60 sec | 1 sec |
| Apnea | Disabled; 20 to 40 sec | 4 set points: Disabled, 20, 30, 40 sec |
| Low Minute Ventilation | Disabled; 1 to 99 L/min | 1 L/min |
| High Rate | 4 to 120 BPM | 1 BPM |
| Low Rate | 4 to 120 BPM | 1 BPM |

Display Ranges & Increments

| Parameters | Control Range | Control Increment |
| --- | --- | --- |
| IPAP | 0 to 50 cm H2O | 1 cm H2O |
| EPAP | 0 to 50 cm H2O | 1 cm H2O |
| CPAP | 0 to 50 cm H2O | 1 cm H2O |
| Rate | 0 to 150 BPM | 1 BPM |
| Exhaled Tidal Volume (VT) | 0 to 4000 ml | 1 ml |
| Minute Ventilation (Min Vent) | 0 to 99 L/min | 1 L/min |
| Total Leak (Tot Leak) | 0 to 300 L/min | 1 L/min |
| Patient Leak(Pt. Leak) | 0 to 300 L/min | 1 L/min |
| Peak Inspiratory Pressure (PIP) | 0 to 50 cm H2O | 1 cm H2O |
| Percent of Patient Triggered Breaths (Pt. Trig) | 0 to 100% | 1% |
| Ti/Ttot | 0 to 100% | 1% |

BiPap® Vision®

Features

Integrated Display Screen
Integrated Display Screen displays easy-to-read real time graphics in waveform or bar scale format, providing the following measured and calculated numerical values to assist with ventilator management:

- IPAP

- Total Breath Rate

- Exhaled Minute Ventilation

- % Patient Triggered Breaths

- Exhaled Tidal Volume

- EPAP

- Ti/Tot

- Peak Inspiratory Pressure

- Patient or Total Leak

Modes

CPAP and S/T are available for maintaining airway patency and providing pressure support ventilation.

- PAV * /T (Proportional Assist Ventilation/Timed mode) helps to promote improved patient comfort and enhanced patient/ventilator synchrony through independent adjustment of Volume and Flow assist based on the patients disease state.

Oxygen Control up to 100% 
Allows you to blend oxygen within the unit in concentrations ranging between 21-100% without auxiliary oxygen tubing attached to the mask. 

Pressure Regulation at the Mask
Pressure regulation at the mask allows you to regulate pressure therapy to assure delivery where it counts (not inside the machine), and automatically adjusts for variations. 

Upgradeable Design Provides Flexibility 
Upgradeable features provide flexibility in your purchase decision, now and in the future. The Vision System gives you the ability to upgrade instead of buying a new machine for each medical revolution. 

BiPAPÂ® Auto-Trak SensitivityTM
Optimum Response Automatically. The innovative development of Auto-Trak SensitivityTM enables the BiPAP Vision System to automatically respond to changing breathing patterns and circuit leaks by identifying leaks and adjusting variable sensitivity thresholds for optimum response with every breath. 

 

Alarm Module. 
Alarm Module gives you a complete range of alarm options. Designed with mask ventilation in mind, the Vision System minimizes nuisance alarms caused by mask leak. When an alarm is activated, the system promptly displays alarm messages on the screen to assist with troubleshooting. Patient alarms include the following:

- High Pressure

- Low Pressure

- Apnea

- Low Minute Ventilation

- High Rate

- Low Rate