 ELECTROMEDICS MEDITEK  PVT. LTD.
   Regd. Office: 303, Sahajanad Palace,  Anandnagar  road, Satellite, Ahmedabad-380015 (Gujarat) Tel.: +91 79 26930184 email : electromedics@gmail.com / sales@electromedics.in

[image: image1.emf]
This product also known as
Patient Monitor, Multi Parameter Monitor, Multipara Monitor, Cardiac Monitor, Portable patient monitor, medical monitor and ICU Patient Monitor.


ECG,RESP,NIBP,SPO2,TEMP,PR/HR
· 12.1″ high resolution color TFT LCD display

· UP to 4 hours working capacity of built_in rechargeable battery

· Anti-ESU, anti- defibrillator

· Large volume of tabular and graphic trends information storage and easy to recall

· Arrhythmic Analysis, Multi_Lead ECG Waveforms Display in Phase, Real time S_T segment analysis, pacemaker detection Drug calculation and titratiotable

· Options of printing, IBP, VGA and EtCO2

· 9 waveforms display

· suitable for adult, pediatric and neonatal patient

· Optional : EtCO2, IBP, Printer, CMS and VGA

Technical specification:
ECG
· Lead Mode: 5 Leads or 3 leads

· Lead selection: I, II, III, avR, avL, avF, V,

· Gain: ´2.5mm/mV, ´5.0mm/mV, ´10mm/mV, ´20mm/mV, auto
Resparation

· Method: Impedance between R-F(RA-LL)

· Apean Alarm: 10 ~ 40 S


SpO2
· Measuring Range: 0 ~ 100 %

· Alarm Range: 0 ~ 100 %

· Resolution: 1 %

· Actualization interval: about 1 Sec.

· Alarm Delay: 10 Sec.


Temperature
· Channel: 2

· Measuring and Alarm Range: 0 ~ 50 °C

· Resolution: 0.1°C

· Accuracy : ±0.1°C


HR and Alarm
· Range
Adult: 15 ~ 300 bpm
Neo/Ped: 15 ~ 350 bpm

· Resolution: 1 bpm

· ST Segment Monitoring Range: Measure and Alarm -2.0 ~ +2.0 mV


NIBP
· Mode: Manual, Auto, STAT

· Pulse Rate Range: 40 ~ 240 bpm

· Alarm Type: SYS, DIA, MEAN

· Resolution Pressure:1mmHg

· Overpressure Protection


Pulse Rate
· Measuring and Alarm Range: 20~300bpm

· Resolution: 1bpm

· Accuracy : ±2bpm

_________________________________________________________________________________     
C.Office :-  J-4 /7, Vighnesh wing, Aditya-Nakoda enclave, Sinhagad road, Pune-411030. (Maharashtra) Tel.:91 2024251401 / 9011016694  email: sales@electromedics.in

